Lab 02: Exploratory Data Analysis (EDA)

1. Objective

The aim of this lab is to:

- Perform Exploratory Data Analysis (EDA) on a real milling dataset.

- Understand the relationship between machining parameters and tool wear.

- Analyze sensor data including:

- Acoustic Emission (AE)

- Vibration

- Current

- Perform:

- Data cleaning

- Missing value handling

- Statistical analysis

- Correlation study

- Outlier detection

- Visualization

- Understand why EDA is critical before applying Machine Learning models.

2. Dataset Description

The dataset used in this lab (mill.csv) represents a real-world milling process monitoring scenario in which tool wear and multiple sensor signals are recorded under different machining conditions. Each row corresponds to a single machining run. The data includes machining parameters such as machining time, depth of cut (DOC), feed rate, and material type, along with tool flank wear (VB), which serves as the primary output variable. In addition, the dataset includes multi-sensor measurements collected during the milling operation, including spindle motor currents (AC and DC), vibration signals measured at both the machine table and spindle, and acoustic emission (AE) signals from the table and spindle locations.

This dataset is designed to simulate an industrial smart manufacturing environment where sensor data is used to monitor tool condition and machine health. The variable VB represents tool flank wear in millimeters and reflects the degradation of the cutting tool over time.

Through this dataset, students will analyze how machining parameters and sensor readings relate to tool wear and understand how data-driven monitoring supports intelligent manufacturing systems.

3. Learning Outcomes

After completing this lab, students will be able to:

- Perform systematic EDA

- Clean real-world noisy data

- Detect missing values and duplicates

- Analyze statistical properties

- Understand feature relationships

- Visualize sensor-based manufacturing data

- Appreciate the importance of EDA in AI/ML pipelines

4. Lab Tasks (Step-by-Step)

Each task must be performed under a separate section header, and every step should be executed in an individual notebook cell.

Task 1: Load the Dataset

Load the dataset and do the following steps.

Steps

- Import necessary libraries

- Load dataset

- Display first and last 5 rows

- Check data types

- Check dataset shape

Task 2: Data Cleaning

Ensure dataset quality before analysis.

Steps

- Check Missing Values

- Fill missing values by selecting the most appropriate method discussed in the provided article.

- Check Duplicate Records

- Remove the Duplicate Records

- Ensure numerical columns are numeric

Task 3: Statistical Analysis

Steps

Students must compute the following:

1. Center (Mean, Median, Mode)

For all numerical columns

📌 Answer the following in a markdown cell:

- Is tool wear normally distributed?

- Is the mean close to median?

2. Spread (Variation)

Calculate:

- Standard deviation

- Variance

- Range (max − min)

- Interquartile Range (IQR = Q3 − Q1)

📌 Answer the following in a markdown cell:

- Which sensor shows highest variability?

- Does vibration vary more than current?

3. Shape (Distribution Pattern)

Analyze distribution using:

- Histogram

- KDE plot

📌 Answer the following in a markdown cell:

- Is distribution symmetric?

- Right skewed (tool wear increasing)?

- Left skewed?

- Multi-modal?

4. Outlier Detection

Use:

- Boxplots

- IQR method

- Z-modified score

After detecting outliers, remove them using a justified method of choice. In a separate Markdown cell, clearly document the selected method, the reasoning behind your choice, the parameters exhibiting significant outliers, and the impact of outlier removal on the dataset’s statistical properties.

Task 4: Correlation Analysis

Steps

Students Must compute:

- Correlation matrix for all numerical features.

- Plot heatmap

📌 Answer the following in a markdown cell:

- Does tool wear correlate strongly with vibration?

- Which sensor is most predictive of tool wear?

- Are some features redundant?

Task 5: Data Visualization

Steps

Students must create:

- Histogram (tool wear)

- Boxplot (sensor comparison)

- Scatter plot (tool wear vs vibration)

- Correlation heatmap

📌 Visualization should answer engineering questions.

5. Engineering Thinking Questions

📌 Answer the following in a markdown cell:

- Which parameter affects tool wear the most?

- Which sensor signal is most stable?

- Are there abnormal machining cycles?

- Can we predict tool wear from sensor data?

- If you were building a ML model, which features would you select?

Submission Requirements

Students must submit:

- Completed notebook with plots

- Written answers in markdown cell